10GBASE-ZR SFP+ SMF 1550nm 120km DDM Transceiver

The SFP+ transceivers are high performance, cost effective modules supporting data rate of 10Gbps and 120km transmission distance with SMF.

The transceiver consists of three sections: a Cooled EML laser transmitter, a APD photodiode integrated with a trans-impedance preamplifier (TIA) and MCU control unit. All modules satisfy class I laser safety requirements.

The transceivers are compatible with SFP Multi-Source Agreement and SFF-8472 digital diagnostics functions.

Product Highlights

Supports 10.3Gbps bit rates

Hot-pluggable SFP+ footprint

1550nm Cooled EML laser and APD photodiode, Up to 120km for SMF transmission

Compliant with SFP+ MSA and SFF-8472 with duplex LC receptacle, compatible with RoHS

Single +3.3V power supply

Real Time Digital Diagnostic Monitoring

Technical specifications

| Vendor Name | OEM | Part Number(s) | HT204-192-EZR-120 |
| --- | --- | --- | --- |
| Transceiver Type | EZR 120km | Reach | 120km |
| Wavelength | 1550nm | Form Type | SFP+ |
| Speed Description | 10Gb/s | Media | SMF |
| Connector | LC Duplex | Extinction Ratio | >8.2 dBm |
| TX Power | 1~6 dBm | Receiver Sensitivity | <-25 dBm |
| Applications | 10GBASE-ZR at 10.3125Gbps 10GBASE-ZW at 9.953Gbps Other Optical links | Environment | Operating: 0 to 70°C Storage: -40°C to 85°C |